Parameter Table of 9JQL Vertical Traction Type Total Mixed Ration Preparer

| - project | - Details | Standard configuration parameters |
| --- | --- | --- |
| - Tank body | Model Specifications | - 9JQL-6 cubic meters |
|  | Box thickness and material | - 6mm/Q355B manganese plate |
|  | Base plate thickness and material | - 16mm/Q355B manganese plate |
|  | - Tank process | CNC bending |
|  | Box filling port | - Open |
| - Stirring section | Number of augers | - 1 |
|  | Spiral Dragon Form | - Vertical |
|  | Spiral material | - Q355B manganese steel plate or NM360 wear-resistant steel plate |
|  | blade thickness | - ≥16mm |
|  | Blade material | - Tungsten carbide coating |
|  | Number of blades | - 3 major |
|  | Blade thickness | - ≤10mm |
|  | Blade lifespan | - ≥1500 hours |
|  | Drive shaft (CM) | - 1850mm |
|  | Each work time | 20 minutes |
|  | Mixing time | 10 minutes |
|  | Mixing uniformity | Pennsylvania screening test passed |
|  | Production efficiency (KG/H) | 2400kg/h-3000kg/h |
| - Powertrain | - Matching power (horsepower) | 60-80 horsepower tractors |
|  | Number of tires | - 4 |
|  | Tire specifications | - 650-16 |
|  | - Gearbox specifications and models | R-series hardened tooth surface planetary reducer / XHG05-51000.01 |
|  | - Gearbox Brands | Imported Kangmai or domestically produced Jiangling |
|  | - Two-stage reducer brands | Jiangling |
|  | - Gearbox Model | XHG07-51000.00 |
|  | - Axle Brand | Adil |
| - Weighing section | Weighing system | - Imported Italian DG/American Avettron 5T/4100 |
|  | Weighing sensor | - 3-point support |
|  | Sensor maximum range | - 15T |
|  | protection level of weighing sensor | - IP67 |
|  | Weighing indicator control method | - Fully automatic |
|  | Measurement accuracy | - 1/3000 |
|  | Weighing indicator voltage | - 12V |
| - Weight and size | Total weight (kg) | ≥ 2160kg |
|  | Overall machine dimensions (mm) | ≥4100*2000*2400mm |
| - Customization range | Box thickness, discharge port direction, iron removal at the discharge port, automatic oiling, paint color, trademark marking, and external display screen for weighing. |  |
| - Note: Mechanical transmission is not allowed when using electric power; they must be separated (either mechanical only or electric only). Tractor output requirement: 540 r/min |  |  |