9JQL Vertical Traction TMR Parameter Table

| - project | - Details | Standard configuration parameters |
| --- | --- | --- |
| - move - force - department - point | - Model Specifications | - 9JQL-36 |
|  | - Supporting power | - 280 horsepower - 360 horsepower |
|  | - speed reducer | - R-series hardened gear planetary reducers |
|  | - Gearbox Brands | - Imported Kangmaier products, domestically produced Jiangling products |
|  | - model | - XHG05-51000.01 |
|  | - Two-stage reducer | - Jiangling |
|  | - model | - XHG07-51000.03 |
| - all - machine - and - stir - mix - department - point | - Spiral Dragon Form | - Vertical |
|  | - Number of augers | - 3 |
|  | - Spiral material | - Q355B manganese steel plate or NM360 wear-resistant steel plate |
|  | - Spiral speed | - 22-29 rpm |
|  | - blade thickness | - 16mm |
|  | - Blade material | - Tungsten carbide alloy |
|  | - Number of blades | - 21st CPC |
|  | - Blade thickness | - ≤10mm |
|  | - Blade lifespan | - ≥1500 hours |
|  | - Drive shaft (CM) | - 1850mm |
|  | - Box thickness and material | - 10mm Q355B manganese plate |
|  | - Base plate thickness and material | - 20mm Q355B manganese plate |
|  | - Box filling method | - Open |
|  | - Stirring speed | - 18,000 tons/hour |
|  | - Each work time | - 25 minutes |
|  | - Mixing time | - 10 minutes |
|  | - Mixing uniformity | - Binzhou screening test passed |
|  | - Forage cutting length | - The length of the forage can vary from 2-6cm depending on the mixing time. |
|  | - Number of tires | - 8 |
|  | - Tire specifications | - Tire size 295/60R22.5 |
| - Weighing section | - Weighing system | - Imported Italian DG/American Aesthetica 7.5T |
|  | - Weighing sensor | - 4 support points |
|  | - Protection rating of weighing indicator | - IP67 |
|  | - Weighing indicator control method | - Fully automatic |
|  | - Sensor maximum range | - 30T |
|  | - Measurement accuracy | - 1/3000 |
|  | - Weighing indicator voltage | - 12V |
| - Discharge section | - Control method | - Hydraulic manual control valve |
|  | - Number of doors opened | - bilateral |
|  | - Production efficiency (KG/H) | - 14400-18000 kg/h |
|  | - Total weight (kg) | - (kg) |
|  | - Overall machine dimensions (mm) | - Approximately 9800*2550*3300 mm, subject to actual production specifications. |
|  | - Note: Mechanical transmission is not allowed when using electric power; they must be separated (either mechanical or electric only). Tractor output requirement: 540 r/min |  |